Sector Guidance for development of CFP-PCR

C-06-01: Annex

“Industrial products (other than foods)”

[About this document]

This Sector Guidance aims to be used when developing a draft CFP-PCR in the product category of “Industrial products (other than foods)”. In development of this Guidance, common items were quoting from existing CFP-PCR which had been developed under the Japanese CFP Pilot Project, and they are consolidated, and they were harmonized with “Requirements for CFP quantification and declaration” and “Requirements for CFP-PCR” under this CFP Program. Therefore, this Guidance includes contents which are not necessarily consistent with existing CFP-PCR under the CFP Pilot Project.

PA-AC-02: Power Detergent

PA-AF-04: Pre-Sensitized Plates for Lithographic Printing

PA-AK-03: Office Furniture

PA-AO-03: Uniform

PA-BA-02: Fire Extinguisher

PA-BK-02: Reuse Battery

PA-CB-01: Wood-plastic Composite

PA-CI-01: IT Equipments

PA-CL-01: Floor Coverings

PA-CU-01: Industrial Filter

[How to use this Sector Guidance]

The businesses which will newly develop a draft CFP-PCR of “Industrial products (other than foods)” shall use in principle this Sector Guidance as a template. In development of a draft CFP-PCR, requirements specific to a target product are required, thereby please confirm whether your draft conforms to “Requirements for CFP quantification and declaration” and “Requirements for CFP-PCR” which describe relative requirements.

(Note)

- Please submit your draft CFP-PCR, remaining your “change history” of MS-Word in the document.

- Italic types in the table refer to explanations of the contents which should be described in CFP-PCR,

for your information. Please delete all the italic types when you submit a draft CFP-PCR.

- For procedures for PCR development, please refer to “procedures for approval of CFP-PCR”.

Carbon Footprint of Products - Product Category Rule of

“Name of CFP-PCR”

This document prescribes the rules on CFP quantification and declaration for “industrial products (other than foods)” under the “CFP Communication Program” (hereinafter called “the CFP Program”) operated and managed by JEMAI (Japan Environmental Management Association for Industry).

The businesses shall conduct CFP quantification and declaration based on this document and “Requirements for CFP quantification and declaration”.

| No. | Items | Contents |
| --- | --- | --- |
| 1 | Scope | This CFP-PCR prescribes rules, requirements, and instructions for CFP quantification and declaration applicable to “Industrial products (other than foods)” under the CFP Program. For the contents which possibly violate laws/regulations related to a target product, compliance of the laws/regulations shall take precedence. When there is an overlap between the scopes of this CFP-PCR and existing CFP-PCR, the reason you will prescribe this CFP-PCR shall be described. |
| 2 | Definitions of product category |  |
| 2-1 | Product category | Industrial products (other than foods) shall be covered. |
| 2-2 | Functions | Functions shall be prescribed by referring to Annex C (informative). |
| 2-3 | Calculation unit (functional unit) | Functional unit shall be prescribed by referring to Annex C (informative). |
| 2-4 | Components of product | The following components shall be included. - Industrial product (product itself and containers/packaging), and accessories Containers/packaging shall refer to the one to be reached the businesses who will use them, regardless of individual, inner, or outer packaging. Accessories shall refer to the one to be reached the businesses who will use them, and always be attached to or supplied with a product. - Transport materials used in each life cycle stage, and indirect materials |
| 3 | Referenced standards and CFP-PCR | The following CFP-PCR shall be referred. - PA-BB: Paper Containers, Packaging and Wrapping (Intermediate goods) - PA-BC: Plastic Containers and Packaging - PA-BD: Metallic Containers and Packaging (Intermediate goods) - PA-BE: Glass Container (Intermediate goods) These four CFP-PCR related to containers/packaging are hereinafter collectively called “the CFP-PCR of containers/packaging”. In this Annex, excerpts from these CFP-PCR are used in No.6-7 and No.11-4. |
| 4 | Terms and definitions | (1) Estimated use period Estimated use period refers to a time period to be quantified the impact of the use and maintenance stage, in quantification of GHG emissions over a life cycle of a product. It shall be set by referring to product catalogue, product specifications, product lifetime or warranty/replacement period stipulated by applicable laws/regulations, or statutory durable period of depreciation, etc. For other terms unfamiliar to the public, or terms which require setting of specific definitions, they shall be set in each CFP-PCR. |
| 5 | Product system (data collection range) |  |
| 5-1 | Product system (data collection range) | The following life cycle stages shall be covered. - The raw material acquisition stage - The production stage - The distribution stage - The use and maintenance stage - The disposal and recycling stage For a process whose data is difficult to be collected respectively from the raw material stage and the production stage, it may be integrated into either of the stage for quantification. For intermediate goods, when it is possible to know actual conditions and information on the use and maintenance stage and the disposal and recycling stage, those stages may be included in the scope of quantification. When an entire life cycle stages are included, its calculation results show “CFP”. When certain life cycle stage is excluded, its calculation results show “partial CFP”. |
| 5-2 | Cut-off criteria and cut-off target | [Stage, process, and flow, to be covered as cut-off target] - Impact other than when using capital goods such as facility for product production - Impact of construction (e.g., construction of production plant, etc.) - Impact of durable goods used for multiple years - Impact of containers/packaging and transport materials which are used for procuring inputs from outside - Of indirect materials, impact of versatile items (e.g., masks, work gloves). - Impact of indirect departments (e.g., clerical division, research division, etc.) - Impact related to the use and maintenance stage when valid scenario cannot be modeled - Impact of land use change [Exceptions of cut-off criteria] When prescribing original cut-off criteria within a scope which conforms to “Requirements for CFP quantification and declaration,” it shall be clearly described in each CFP-PCR. |
| 5-3 | Life cycle flow chart | General life cycle flow chart is shown in Annex A (normative). When quantifying CFP, specific life cycle flow chart detailed for each target product shall be created, within a scope which is not deviating from the life cycle flow chart. |
| 6 | CFP quantification method applied to all stages |  |
| 6-1 | Range of primary data collection | Data collection range of primary data shall be described in No.7-2, No.8-2, No.9-2, No.10-2, and No.11-2. For data collection items outside of the range of primary data collection, primary data may be collected as appropriate. |
| 6-2 | Quality of primary data | Not stipulated. When originally prescribing quality of primary data within a scope which conforms to “Requirements for CFP quantification and declaration,” it shall be clearly described in each CFP-PCR. |
| 6-3 | Primary data collection method | Not stipulated. When originally prescribing primary data collection method within a scope which conforms to “Requirements for CFP quantification and declaration,” it shall be clearly described in each CFP-PCR. |
| 6-4 | Quality of secondary data | Not stipulated. When originally prescribing quality of secondary data within a scope which conforms to “Requirements for CFP quantification and declaration,” it shall be clearly described in each CFP-PCR. |
| 6-5 | Secondary data collection method | Not stipulated. When originally prescribing secondary data collection method within a scope which conforms to “Requirements for CFP quantification and declaration, it shall be clearly described in each CFP-PCR.” For example, secondary data to be used can be specified in each CFP-PCR. |
| 6-6 | Allocation | [Rules on criteria of allocation] Not stipulated. When originally prescribing allocation method within a scope which conforms to “Requirements for CFP quantification and declaration, it shall be clearly described in each CFP-PCR.” [Rules on avoidance of allocation] Not stipulated. For an important process which requires allocation, method of avoiding allocation shall be prescribed as appropriate. [Rules on target of allocation] Not stipulated. For an important process which requires allocation, it shall be prescribed as appropriate whether such individual processes are conducted allocation or not. |
| 6-7 | Scenario | [Collection of data on transport] When it is difficult to collect primary data on transport volume (or on fuel consumption amount), and when no scenario is set for each stage, the scenario in Annex B (normative) shall be used. [Wastes] For treatment method, when it is difficult to collect primary data or when it is not set a scenario for each stage, the following assumptions shall be used for quantification: the materials which can be incinerated (e.g., paper, plastics) are assumed to be treated by incineration; the materials which cannot be incinerated (e.g., metals) are assumed to be treated by landfill. For the items covered by “the CFP-PCR of containers/packaging,” the scenario of wastes treatment prescribed in “the CFP-PCR of containers/packaging” may be applied. |
| 6-8 | Other | Not stipulated. When originally prescribing a series product, it shall be prescribed in each CFP-PCR. |
| 7 | Requirements for the raw material acquisition stage |  |
| 7-1 | Range of the processes | (1) Process related to production and transport of “important raw materials” (2) Process related to production and transport of “other raw materials” (3) Process related to production and transport of “containers and packaging” and “accessories” |
| 7-2 | Data collection items | The data items listed in the following table shall be collected. (1) Process related to production and transport of “important raw materials” Activity; Category of activity; Emission factor to be multiplied by activity. “Components of raw materials” Amount of each component amount needed for production of raw materials to be input to product production site; Primary; “Each component” Emission factor of production. “Components of raw materials” Transport volume (or fuel consumption amount) of raw materials to production site; *1; “Each transport mean” Emission factor of transport. “Water” “Fuels” “Electricity” Input amounts of raw materials to production process; Primary; “Water” “Fuels” “Electricity” Emission factor of production, supply, and use. “Indirect materials (production materials, chemicals, etc.)” Input amounts of raw materials to production process; Primary; “Indirect materials (production materials, chemicals, etc.)” Emission factor of production. “Indirect materials (production materials, chemicals, etc.)” Transport volume (or fuel consumption amount) to production site of raw materials; *1; “Each transport mean” Emission factor of transport. “Wastes” “Waste water” *2. “Important raw materials” Transport volume (or fuel consumption amount) of items to product production site; *1; “Each transport mean” Emission factor of transport. (2) Process related to production and transport of “other raw materials” Activity; Category of activity; Emission factor to be multiplied by activity. “Other raw materials” Input amount to product production site; Primary; “Each part and material” Emission factor of production. “Other raw materials” Transport volume (or fuel consumption amount) of items to product production site; *1; “Each transport mean” Emission factor of transport. (3) Processes related to production and transport of “containers and packaging” and “accessories” Activity; Category of activity; Emission factor to be multiplied by activity. “Containers and packaging” “Accessories” Input amount to product production site; Primary; “Containers and packaging” “Accessories” Emission factor of production. “Containers and packaging” “Accessories” Transport volume (or fuel consumption amount) of items to product production site; *1; “Each transport mean” Emission factor of transport. *1. The following items shall be collected as primary data. [The fuel consumption method] - “Fuel consumption” for each transport mean [The fuel cost method] - “Fuel cost” for each transport mean - “Transport distance” for each transport mean [The ton-kilometer method] - “Transport load” for each transport mean *2. Data collection items related to wastes and waste water Activity; Category of activity; Emission factor to be multiplied by activity. “Wastes” “Waste water” Emission for each treatment method; Primary or scenario; “Each treatment method” Emission factor of treatment. “Wastes” Transport volume (or fuel consumption amount) to each treatment facility; *1; “Each transport mean” Emission factor of transport. “Of wastes, component derived from fossil resource” Incineration volume of the component; Primary or scenario; “Each component derived from fossil resource” Emission factor of incineration. “Of wastes, organic component” Landfill volume of the component; Primary or scenario; “Each organic component” Emission factor of anaerobic decomposition. |
| 7-3 | Primary data collection method and requirements | Not stipulated. |
| 7-4 | Scenario | Not stipulated. |
| 7-5 | Other | Not stipulated. |
| 8 | Requirements for the production stage |  |
| 8-1 | Range of the processes | (1) Production process of industrial product itself (processing, assembly, inspection, storage, and packing, etc.) (2) Transport process between sites |
| 8-2 | Data collection items | The data items listed in the following table shall be collected. (1) Production process of industrial product itself (processing, assembly, inspection, storage, and packing, etc.) Activity; Category of activity; Emission factor to be multiplied by activity. “Water” “Fuels” “Electricity” Input amounts to product production process; Primary; “Water” “Fuels” “Electricity” Emission factors of production, supply, and use. “Indirect materials (chemicals, and materials for production, inspection, storage, packing, and chemicals, etc.)” Input amounts to product production process; Primary; “Each indirect materials” Emission factor of production. “Indirect materials (chemicals, materials for production, inspection, storage, packing, and chemicals, etc.)” Transport volume (or fuel consumption amount) of items to product production site; *1; “Each transport mean” Emission factor of transport. “Wastes” “Waste water” *2. (2) Transport process between sites Activity; Category of activity; Emission factor to be multiplied by activity. “Indirect materials (transport materials)” Input amounts to transport process between sites; Primary; “Each indirect materials” Emission factor of production. “Indirect materials (transport materials)” Transport volume (or fuel consumption amount) of items to product production site; *1; “Each transport mean” Emission factor of transport. “Items transported” Transport volume (or fuel consumption amount) of items between each site; *1; “Each transport mean” Emission factor of transport. “Wastes” *2. *1. Requirements for transport volume (or fuel consumption amount) shall conform to No.7-2. *2. Requirements for wastes and waste water shall conform to No.7-2. [Primary data collection items to be collected for allocation] - Production amount of “industrial product itself” - Production amount of “co-product” |
| 8-3 | Primary data collection method and requirements | Not stipulated. |
| 8-4 | Scenario | Not stipulated. |
| 8-5 | Other | Not stipulated. |
| 9 | Requirements for the distribution stage |  |
| 9-1 | Range of the processes | (1) Transport/storage process of “shipped items” Storage process may be excluded from the assessment when it has small contributions to an entire life cycle of a product. (2) Sales process Sales process may be excluded from the assessment when it has small contributions to an entire life cycle of a product. |
| 9-2 | Data collection items | The data items listed in the following table shall be collected. (1) Transport/storage process of “shipped items” Activity; Category of activity; Emission factor to be multiplied by activity. “Shipped items” Transport volume (or fuel consumption amount); *1; “Transport mean” Emission factor of transport. “Water” “Fuels” “Electricity” Transport volume of shipped items, and input amounts to transport/storage process; Secondary or scenario; “Water” “Fuels” “Electricity” Emission factors of production, supply, and use. “Indirect materials (transport materials)” Transport volume of shipped items, and input amounts to transport/storage process; Secondary or scenario; “Indirect materials (transport materials)” Emission factor of production. “Indirect materials (transport materials)” Transport volume of shipped items, and transport volume to storage site (or fuel consumption amount); *1; “Transport mean” Emission factor of transport. “Wastes” “Waste water” *2. (2) Sales process Activity; Category of activity; Emission factor to be multiplied by activity. “Product” Sales amount for each sales method (at store, by delivery); Secondary or scenario; “Each sales method” Emission factor of sales. “Water” “Fuels” “Electricity” Input amounts to sales process; Secondary or scenario; “Water” “Fuels” “Electricity” Emission factors of production, supply, and use. *1. Requirements for transport volume (or fuel consumption amount) shall conform to No.7-2. *2. Requirements for wastes and waste water shall conform to No.7-2. |
| 9-3 | Primary data collection method and requirements | Not stipulated. |
| 9-4 | Scenario | [Rules on sales amount for each sales method] The businesses shall set a valid scenario of sales amount for each sales method. |
| 9-5 | Other | Not stipulated. |
| 10 | Requirements for the use and maintenance stage |  |
| 10-1 | Range of the processes | (1) Process of install/construction of product This process shall be covered when energy or consumables are required for install or construction of the product. (2) Process of use/maintenance This process shall be covered when energy or consumables are required for use or maintenance. |
| 10-2 | Data collection items | The data items listed in the following table shall be collected. (1) Process of install/construction of product Activity; Category of activity; Emission factor to be multiplied by activity. “Water” “Fuels” “Electricity” Input amounts to the process of install/construction of product; Primary or scenario; “Water” “Fuels” “Electricity” Emission factors of production, supply, and use. “Install/construction materials” Input amounts to the process of install/construction of product; Primary or scenario; “Install materials” Emission factor of production. “Install/construction materials” Transport volume (or fuel consumption amount) from production site to user; *1; “Transport mean” Emission factor of transport. “Wastes (losses from product install, install materials, etc.)” “Waste water” *2. (2) Process of use/maintenance Activity; Category of activity; Emission factor to be multiplied by activity. “Water” “Fuels” “Electricity” Input amounts during estimated use period; Primary or scenario; “Water” “Fuels” “Electricity” Emission factors of production, supply, and use. “Consumables” Input amounts during estimated use period; Primary or scenario; “Consumables” Emission factor of production. “Consumables” Transport volume (or fuel consumption amount) from production site to user; *1; “Transport mean” Emission factor of transport. “Wastes (consumables) ” “Waste water” *2. *1. Requirements for transport volume (or fuel consumption amount) shall conform to No.7-2. *2. Requirements for wastes and waste water shall conform to No.7-2. |
| 10-3 | Primary data collection method and requirements | Collection method and requirements shall be prescribed in each CFP-PCR. |
| 10-4 | Scenario | [Rules on scenario used for quantifying impacts of use process and maintenance process] For any standard install/construction method or standard use method is described in a product or a product catalogue, scenario shall be developed based on the method. When such method does not exist, valid scenario shall be developed in each CFP-PCR. [Rules on data of electricity consumption] For each consumption amount of a product requiring water, electricity, and/or fuels at the time of use, rate values described in a catalogue, etc., or actual values stipulated by applicable laws/regulations and standards shall be collected and used. When such information cannot be obtained, a valid scenario shall be developed in each CFP-PCR. |
| 10-5 | Other | Not stipulated. |
| 11 | Requirements for the disposal and recycling stage |  |
| 11-1 | Range of the processes | (1) Disposal and recycling process of “used product” (2) Disposal and recycling process of “waste containers/packaging, accessories” |
| 11-2 | Data collection items | The data items listed in the following table shall be collected. (1) Disposal and recycling process of “used product” Activity; Category of activity; Emission factor to be multiplied by activity. “Used product” Emissions for each treatment method; Primary or scenario; “Each treatment method” Emission factor. “Used product” Transport volume (or fuel consumption amount) to each treatment facility; *1; “Each transport mean” Emission factor. “Of used product, component derived from fossil resource” Incineration volume of the component; Primary or scenario; “Incineration of each component derived from fossil resource” Emission factor. “Of used product, organic component” Landfill volume of the component; Primary or scenario; “Each organic component” Emission factor of anaerobic decomposition. (2) Disposal and recycling process of “waste containers/packaging, accessories” Activity; Category of activity; Emission factor to be multiplied by activity. “Waste containers/packaging, accessories” Emissions for each treatment method; Primary or scenario; “Each treatment method” Emission factor of treatment. “Waste containers/packaging, accessories” Transport volume (or fuel consumption amount) to each treatment facility; *1; “Each transport mean” Emission factor. “Of waste containers/packaging and accessories, component derived from fossil resource” Incineration volume of the component; Primary or scenario; “Incineration of each component derived from fossil resource” Emission factor. “Of waste containers/packaging and accessories, organic resource” Landfill volume of the resource; Primary or scenario; “Each organic resource” Emission factor of anaerobic decomposition. *1 Requirements for transport volume (or fuel consumption amount) shall conform to No.7-2. |
| 11-3 | Primary data collection method and requirements | Not stipulated. |
| 11-4 | Scenario | [Rules on treatment method of wastes] For ratio of treatment method of wastes, the following scenario shall be used. - For disposal treatment scenario of “waste containers/packaging,” the scenario of “the CFP-PCR of containers/packaging” - For disposal treatment scenario of “used product,” the scenario based on investigation report, etc. conducted by a country or public entity. However, these reports should be appropriate investigation range and the latest investigation period. Scenario should be prescribed in each CFP-PCR. |
| 11-5 | Other | Not stipulated. |
| 12 | CFP declaration method |  |
| 12-1 | Additional information | Not stipulated. Refer to No.3.2.5 in “Requirements for CFP quantification and declaration”. |
| 12-2 | Registration information | [Rules on required contents] The following item shall be displayed. - Quantification method of the use and maintenance stage (scenario) Refer to No.3.2.6 in “Requirements for CFP quantification and declaration”. |
| 12-3 | Other | Not stipulated. |

Annex A (normative): Life cycle flow chart

(1) Raw materials acquisition stage

Production of important raw materials

Transport/storage process

Transport of wastes

(2) Production stage

(3) Distribution stage

(4) Use and maintenance stage

(5) Disposal/recycling stage

Stores

Treatment

Wastes

Transport

Raw materials

Use

Install

System Boundary

Treatment

Other materials production

Transport of

raw materials

Transport of containers/packaging and accessories

Production of containers/packaging and accessories

Raw materials

Containers/packaging, accessories

Product

Wastes

Transport

Processing/assembling process

Inspection, storage, and packing process

Treatment

Waste water

Transport of

indirect materials

Indirect materials production

Indirect materials

Treatment

Transport of

indirect materials

Indirect materials production

Indirect materials

Transport of

raw materials

Wastes

Transport

Treatment

Transport of materials

/consumables

Materials/consumables production

Materials/consumables

* All processes related to supply and use of energy and water are omitted from this flow chart.

Item/material covered

* This flow chart shows an overview of a representative life cycle flow chart of industrial products. For CFP quantification of a specified product, it shall be quantified according to processes actually used, such as by omitting unneeded process from this chart.

[Legend]

Process covered

Process not covered

# Annex B (normative): Transport scenario

The following shows transport scenarios when no primary data can be collected.

B1. Transport distance

- Transport within a city or not across adjacent cities: 50km

- Transport within a prefecture: 100km

- Transport possibly across prefectural border to another side of the border: 500km

- Transport which is not limited within a specific area (domestic): 1,000km

- Road transport distance within overseas country: 500km

- Sea transport distance between ports (port => port)

B2. Transport means and loading ratio

| Life cycle flow chart | Scenario |  |
| --- | --- | --- |
| The raw material acquisition stage - Transport for raw material procurement | Road transport only | <Transport mean> 10-ton truck <Loading ratio> 62% |
|  | Transport including sea transport (Domestic transport in a country from which products will be imported; Production site => Port) | <Transport mean> 10-ton truck <Loading ratio> 62% |
|  | Transport including sea transport (International transport; Port => Port） | <Transport mean> Container ship (<4,000TEU) |
|  | Transport including sea transport (Domestic transport; Port => Client) | <Transport mean> 10-ton truck <Loading ratio> 62% |
| The production stage - Transport between sites - Transport for indirect material procurement - Transport of wastes | Transport between sites | <Transport mean> 2-ton truck <Loading ratio> 58% |
|  | Transport for indirect material procurement | Same as the raw material acquisition stage |
|  | Transport of wastes (Production site => Treatment facility) | <Transport mean> 2-ton truck <Loading ratio> 58% |
| The distribution stage - Transport of products - Transport of wastes | In case of overseas production site (Production site => Port in production country) | <Transport mean> 10-ton truck <Loading ratio> 62% |
|  | In case of overseas production site (Port in production country => Domestic port) | <Transport mean> Container ship (<4,000TEU) |
|  | In case of overseas production site (Domestic port => Store) | <Transport mean> 10-ton truck <Loading ratio> 62% |
|  | In case of domestic production site (Production site => Store) | <Transport mean> 10-ton truck <Loading ratio> 62% |
|  | Transport of wastes (Store => Treatment facility) | <Transport mean> 2-ton truck <Loading ratio> 58% |
| The disposal and recycling stage | Transport of wastes (Garbage collection site => Treatment facility) | <Transport mean> 2-ton truck <Loading ratio> 58% |

Annex C (informative)： Quantification unit, unit to be labeled, and scope of quantification, for each purpose of CFP quantification and declaration

| Product | Purpose of CFP quantification and declaration | Functions | Quantification unit (functional unit) | Unit to be labeled | Scope of quantification (product system) |
| --- | --- | --- | --- | --- | --- |
| Laundry detergent | By calculating CFP (including functions at the time of detergent use), to evaluate improvement of detergent functions, and to communicate the results to stakeholders such as consumers. | Washing of clothes | Washing of 300kg of clothes (since one box includes 300g of detergent, and 5kg of detergent can wash 5kg of clothes) | Per washing (5kg of clothes) | Production of detergent, provision of water for washing, drainage treatment, electricity use by washing machine, etc. |
| Sweet potatoes | By knowing where and how GHG emissions are emitted before reaching potatoes to consumers, to make efforts for improving the results, and to communicate effects by efforts of improvement over time. | Provision of sweet potatoes to ordinary households | Standard weight of sweet potatoes: 1kg | 1kg of sweet potatoes | Cultivation and distribution of sweet potatoes, disposal of peels, etc. |
| Publishing and commercial printing | Aiming to be quoted in a CFP-PCR of published matter (e.g. magazine), to assess up to and including the production stage of the printed matter, not assess an entire life cycle. | Provision of publishing and commercial printing | 5,000 copies of printed materials | 5,000 copies of printed materials | Production of paper, production of printed materials |
| Copier | By knowing contributions of copiers to mitigation of global warming, to improve functions of copiers and to communicate to consumers. | Printing of printing sheets | Printing of 30,000 printing sheets | Per printing (not including paper) | Production and disposal of copier, etc. |
| Copier | By knowing GHG emissions from printed matter, to understand improvement point and to use the information as data source for communicating to stakeholders including paper manufactures. | Provisions of printed materials | Provisions of 30,000 sheets of printed materials | Per printed material (including paper) | Production and disposal of copier, production of paper, disposal of printed materials, etc. |
| Ramen bowl | To show consumers about the small contributions of dishes to global warming compared with ramen, and to communicate importance of foods. | Provision of ramen | Provision of 365 bowls of ramen | Per bowl of ramen (including cooking of ramen) | Production, washing, and disposal of ramen bowl, production and cooking of ramen and ingredients, etc. |
| Ramen bowl | To communicate importance of proper dishwashing to consumers. | Provision of clean ramen bowl | Provision of 365 clean ramen bowls | Per use of a ramen bowl | Production, washing, and disposal of ramen bowl |
| Ramen bowl | Aiming to know the contributions of ramen bowl to mitigation of global warming, to consider environmental friendly ramen bowl, while communicating with consumers. | Provision of ramen bowl | Provision of a ramen bowl | Per ramen bowl | Production and disposal of ramen bowl, etc. |